| Department of Electrical Engineering Assignment Date: 14/04/2020 Course Details |  |  |  |
| --- | --- | --- | --- |
| Course Title: | Signals and Systems | Module: | 6th |
| Instructor: |  | Total Marks: | 30 |

Student Details

| Name: |  | Student ID: |  |
| --- | --- | --- | --- |

| Q1. | (a) | - Find the total solution of the following Linear Constant difference equation by - Homogeneous and Particular solution method - Zero input and Zero State solution method. - After finding total solution plot the responses by putting at least four different values and comment on both methods. - Y[n] + 0.567 Y[n-1] + 33.3Y[n-2] + Y[n-4] = x[n] - For unit step x[n] = 10u[n] - with y[-1] = 1, y[-2] = -1 | Marks 10 |
| --- | --- | --- | --- |
|  |  |  | CLO 1 |
| Q2. | (a) | - Find the sampling frequency of | Marks 05 |
|  |  |  | CLO 1 |
|  | (b) | - Sketch the block diagram representation of discrete-time systems described by the following input-output relation. Also find order of the system, total number of Adders and Scalars. - y[n] – 4 y[n-2] = 3 x[n] + 2 x[n-1] + 4x[n-4] - y[n] – 10.3 y[n-8] = x[n] + 3 x[n-1] | Marks 05 |
|  |  |  | CLO 1 |
| Q3. | (a) | Consider the following two sequences x[n] and y[n]: x[n] = [1, 3, 6, -4,, -2, 1, 3, 0, 0, 3] y[n] = [2, 4, -2, , 2, 0, 0, -2, 5] Sketch and label the following sequences. Also specify either they are RSS, LSS or TSS. | Marks 05 |
|  |  |  | CLO 2 |
|  | (b) | Consider the signal x(t) = (-1, -1, -2, -2, -2, -1) Plot - x(t-2) - x(-t) - 25x(-t) - X(t+5) - X(1/t) | Marks 05 |
|  |  |  | CLO 2 |

*********************************************************************************

GOOD LUCK