| Department of Electrical Engineering Mid-term Assignment Date: 24/08/2020 Course Details |  |  |  |
| --- | --- | --- | --- |
| Course Title: | Digital Signal Processing | Module: | 6th |
| Instructor: | Dr. Rafiq Mansoor | Total Marks: | 30 |

Student Details

| Name: |  | Student ID: |  |
| --- | --- | --- | --- |

| Q1. | (a) | Consider the following analog signal xat=3cos100πt+4sin200πt - Determine the minimum sampling rate required to avoid aliasing. - Suppose that the signal is sampled at the rate Fs=100Hz. What is the discrete-time signal obtained after sampling? Also explain the effect of this sampling rate on the newly generated discrete time signal. - What is the analog signal yat we can reconstruct from the samples if we use ideal interpolation? | Marks 5 |
| --- | --- | --- | --- |
|  |  |  | CLO 1 |
|  | (b) | Consider a discrete time signal which is given by xn=0.5n , n≥00, n<0 This is signal is sampled at the rate Fs=2Hz. - Draw the sampled signal. - The samples of the signals are intended to carry 3 bits per sample. Determine the quantization level and quantization resolution to quantized the sampled signal achieved in part i . - Perform the process of truncation and rounding off on all the values of the sampled signal and find the quantization error for each of the sampled data. Express your answer in tabular form. | Marks 5 |
|  |  |  | CLO 1 |
| Q2. |  | Compute the convolution y(n) of the following signal xn=αn+1, -3≤n≤50, elsewhere hn=2n, 0≤n≤40, elsewhere | Marks 10 |
|  |  |  | CLO 2 |
| Q3. |  | Determine the z- transform of the following signals and also sketch its Region of Convergence (ROC). - xn=(14)n , n≥0(13)-n, n<0 - xn=(12)n-3n, n≥00, elsewhere | Marks 10 |
|  |  |  | CLO 2 |